| Test | Result | Acceptance Limits | Conclusion |
| --- | --- | --- | --- |
| Description | A white, crystalline powder; odorless | A white crystalline powder | Complies |
| Solubility | Complies | Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. | Complies |
| Identification A (IR) | Complies (purity index 0.9979) | IR spectrum concordant with working standard | Complies |
| Identification B (chemical) | Complies | violet → orange → yellow | Complies |
| Appearance of solution | Complies | clear, ≤ YS7 | Complies |
| pH | 6.02 | 5.9 to 6.3 | Complies |
| Heavy metals | < 10 ppm | NMT 10 ppm | Complies |
| Chlorides | < 100 ppm | NMT 100 ppm | Complies |
| Sulphated ash | 0.04 % | NMT 0.1 % | Complies |
| Loss on drying | 0.25 % | NMT 0.5 % | Complies |
| Assay | 99.72 % | 98.5% to 101.5% | Complies |

Complies with the IP specification