| 1.0 |  | Description: |  |
| --- | --- | --- | --- |
| Procedure: Take about 1.0 g of sample and spread to clean Petri dish. Check product visually for colour. Acceptance Criteria: A white, crystalline powder. |  |  |  |
| 2.0 |  | Solubility: |  |
| Procedure: Take about 1.0 g of sample into 10 ml of water and check solubility. Acceptance Criteria: Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. |  |  |  |
| 3.0 |  | Identification: |  |
|  |  | A) By IR Spectrophotometry: |  |
| Acceptance Criteria: IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. |  |  |  |
|  |  | B) By Chemical test: |  |
| Acceptance Criteria: The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. |  |  |  |
| 4.0 |  | Appearance of solution: |  |
| Acceptance Criteria: A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely colored than reference solution YS7. |  |  |  |
| 5.0 |  | pH: |  |
| Acceptance Criteria: 5.9 to 6.3 |  |  |  |
| 6.0 |  | Chlorides: |  |
| Acceptance Criteria: NMT 100 ppm |  |  |  |
| 7.0 |  | Heavy metals: |  |
| Acceptance Criteria: NMT 10 ppm |  |  |  |
| 8.0 |  | Sulphated ash: |  |
| Acceptance Criteria: NMT 0.1% |  |  |  |
| 9.0 |  | Loss on drying: |  |
| Acceptance Criteria: NMT 0.5% |  |  |  |
| 10.0 |  | Assay (On dried basis): |  |
| Acceptance Criteria: 98.5% to 101.5% |  |  |  |
| 11.0 |  | Sieve analysis: |  |
| Acceptance Criteria: As per customer requirement |  |  |  |
| 12.0 |  | Bulk Density: |  |
| Acceptance Criteria: As per customer requirement |  |  |  |
| 13.0 |  | Elemental impurities by AAS (As per ICH Q3D): |  |
|  |  | A) Lead (Pb): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | B) Arsenic (As): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | C) Mercury (Hg): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.1 µg/g |  |  |  |
|  |  | D) Cadmium (Cd): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | E) Cobalt (Co): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | F) Vanadium (V): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | G) Nickle (Ni): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | H) Lithium (Li): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | I) Antimony (Sb): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | J) Copper (Cu): |  |
| Acceptance Criteria: Lowest detection limit NMT 0.5 µg/g |  |  |  |
|  |  | K) Foreign matter: |  |
| Acceptance Criteria: Should be absent in 5 gm sample |  |  |  |
|  |  | L) Ethylene oxide: |  |
| Acceptance Criteria: NMT 0.1 ppm |  |  |  |
|  |  | M) OVI/Residual solvent: |  |
| Acceptance Criteria: As per ICH guideline Q3C(R8) |  |  |  |
| 14.0 |  | Bacterial endotoxin test: |  |
| Acceptance Criteria: Lowest detection limit 5 EU/g |  |  |  |
| 15.0 |  | Pathogen in general: |  |
| Acceptance Criteria: Should be absent |  |  |  |
| 16.0 |  | TAMC: |  |
| Acceptance Criteria: NMT 10³ CFU/ g |  |  |  |
| 17.0 |  | TYMC: |  |
| Acceptance Criteria: NMT 10² CFU/ g |  |  |  |
| 18.0 |  | Escherichia coli: |  |
| Acceptance Criteria: Should be absent |  |  |  |
| 19.0 |  | Salmonella: |  |
| Acceptance Criteria: Should be absent |  |  |  |
| 20.0 |  | Pseudomonas aeruginosa: |  |
| Acceptance Criteria: Should be absent |  |  |  |
| 21.0 |  | Staphylococcus aureus: |  |
| Acceptance Criteria: Should be absent |  |  |  |
| 22.0 |  | Coliforms: |  |
| Acceptance Criteria: Should be absent |  |  |  |

Revision History:

| Specification No. | Revision Made | Ref. CC No. | Effective Date |
| --- | --- | --- | --- |
| SPEC/FG00038/01(R-00) | New specification | ------- | 21 APR 2021 |
| SPEC/FG00038/01(R-01) | Format revised as per revised SOP QC/12.Some contents revised as per FSSAI requirement. | CC/2022/003 | 01 JUN 2022 |
| SPEC/FG00038/01(R-02) | Elemental impurity by AAS added. Ethylene oxide test added. | CC/2023/010 | 01 JUN 2023 |