| Mfg. Date | : | 01 FEB 2026 | Exp. Date | : | 31 JAN 2031 |
| --- | --- | --- | --- | --- | --- |
| Test Request Sheet No. | : | None | A.R. No. | : | 2026 GCN 09 |
| Batch Size | : | 3000 Kgs | Quantity Sampled | : | None |
| Received Date | : | 02 FEB 2026 | Testing Date | : | 03 FEB 2026 |
| Reference Specification/MOA No. | : | SPEC/FG00038/01 (IP)MOA/FG00038/01 (IP) | Issuance Stamp by QA |  |  |
| Completion Date | : | 05 FEB 2026 |  |  |  |

| Sr. No | Tests |  | Results | Limits |
| --- | --- | --- | --- | --- |
| 1. | Description |  | A white, crystalline powder; odorless | A white crystalline powder |
| 2. | pH |  | 6.02 | 5.9 to 6.3 |
| 3. | Heavy metals |  | < 10 ppm | NMT 10 ppm |
| 4. | Loss on drying |  | 0.25 % | NMT 0.5 % |
| 5. | Assay |  | 102.1 % | 98.5% to 101.5% |

Note: Above mentioned product complies / does not comply as per IP specification.

| 1 | Description: |  |
| --- | --- | --- |
| Procedure: Examine the sample visually under adequate light. Readings: White crystalline powder; odorless Result: A white, crystalline powder; odorless Acceptance criteria: A white crystalline powder Conclusion: Satisfactory Analyzed By: Twinkal Tarapara Checked By: Dipak Prajapati |  |  |

| 2 | pH: |  |
| --- | --- | --- |
| Instrument Used: pH Meter MET-PH-04 (Cal. due 15 JAN 2027) Procedure: Dissolve 1.0 g in 10 mL of carbon dioxide-free water. Measure pH. Readings: pH meter reading: 6.02 Result: 6.02 Acceptance criteria: 5.9 to 6.3 Conclusion: Satisfactory Analyzed By: Twinkal Tarapara Checked By: Dipak Prajapati |  |  |

| 3 | Heavy metals: |  |
| --- | --- | --- |
| Reagent Used: Lead standard solution, Ammonia solution Procedure: Proceed as per MOA section for Heavy metals. Readings: Sample solution colour: lighter than standard Result: < 10 ppm Acceptance criteria: NMT 10 ppm Conclusion: Satisfactory Expiry Acknowledgement: Expired reagent lot RE-2024-11 used with QA deviation DEV-2026-014 acknowledged. Analyzed By: Twinkal Tarapara Checked By: Dipak Prajapati |  |  |

| 4 | Loss on drying: |  |
| --- | --- | --- |
| Instrument Used: Hot air oven HO-12 Procedure: Dry 1.0 g at 105 °C to constant weight. Readings: Initial: 1.0020 g; Final: 0.9995 g Calculated Result: 0.25 % Result: 0.25 % Acceptance criteria: NMT 0.5 % Conclusion: Satisfactory Analyzed By: Twinkal Tarapara Checked By: Dipak Prajapati |  |  |

| 5 | Assay: |  |
| --- | --- | --- |
| Instrument Used: Auto titrator AT-07 Reagent Used: 0.1 M Perchloric acid, Crystal violet Procedure: Titrate with 0.1 M perchloric acid using crystal violet indicator. Readings: Titre: 8.42 mL; Blank: 0.12 mL Calculated Result: 102.1 % Result: 102.1 % Acceptance criteria: 98.5% to 101.5% Conclusion: Not satisfactory OOS Acknowledgement: OOS investigated per SOP-QC-012; batch disposition pending QA review. Analyzed By: Twinkal Tarapara Checked By: Dipak Prajapati |  |  |