| Mfg. Date | : |  | Exp. Date | : |  |
| --- | --- | --- | --- | --- | --- |
| Test Request Sheet No. | : |  | A.R. No. | : |  |
| Batch Size | : |  | Quantity Sampled | : |  |
| Received Date | : |  | Testing Date | : |  |
| Reference Specification/MOA No. | : | SPEC/FG00038/01 (IP)MOA/FG00038/01 (IP) | Issuance Stamp by QA |  |  |
| Completion Date | : |  |  |  |  |

| Sr. No | Tests |  | Results | Limits |
| --- | --- | --- | --- | --- |
| 1. | Description |  |  | A white, crystalline powder. |
| 2. | Solubility |  |  | Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. |
| 3. | Identification |  |  |  |
|  | By IR Spectrophotometry |  |  | IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. |
|  | By Chemical test |  |  | The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. |
| 4. | Appearance of solution |  |  | A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely colored than reference solution YS7. |
| 5. | pH |  |  | 5.9 to 6.3 |
| 6. | Chlorides |  |  | NMT 100 ppm |
| 7. | Heavy metals |  |  | NMT 10 ppm |
| 8. | Sulphated ash |  |  | NMT 0.1% |
| 9. | Loss on drying |  |  | NMT 0.5% |
| 10. | Assay (On dried basis) |  |  | 98.5% to 101.5% |
|  | Additional Tests |  |  |  |
| 11. | Sieve analysis |  |  | As per customer requirement |
| 12. | Bulk Density |  |  | As per customer requirement |
| 13. | Elemental impurities by AAS (As per ICH Q3D) |  |  |  |
|  | Lead (Pb) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Arsenic (As) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Mercury (Hg) |  |  | Lowest detection limit NMT 0.1 µg/g |
|  | Cadmium (Cd) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Cobalt (Co) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Vanadium (V) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Nickle (Ni) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Lithium (Li) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Antimony (Sb) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Copper (Cu) |  |  | Lowest detection limit NMT 0.5 µg/g |
|  | Foreign matter |  |  | Should be absent in 5 gm sample |
|  | Ethylene oxide |  |  | NMT 0.1 ppm |
|  | OVI/Residual solvent |  |  | As per ICH guideline Q3C(R8) |
|  | Microbiological parameters |  |  |  |
| 14. | Bacterial endotoxin test |  |  | Lowest detection limit 5 EU/g |
| 15. | Pathogen in general |  |  | Should be absent |
| 16. | TAMC |  |  | NMT 10³ CFU/ g |
| 17. | TYMC |  |  | NMT 10² CFU/ g |
| 18. | Escherichia coli |  |  | Should be absent |
| 19. | Salmonella |  |  | Should be absent |
| 20. | Pseudomonas aeruginosa |  |  | Should be absent |
| 21. | Staphylococcus aureus |  |  | Should be absent |
| 22. | Coliforms |  |  | Should be absent |

Note: Above mentioned product complies / does not comply as per IP specification.

|  | Analyzed By | Checked By | Approved By |
| --- | --- | --- | --- |
| Sign |  |  |  |
| Date |  |  |  |

| 1.0 | Description: |  |
| --- | --- | --- |
| Procedure: Take about 1.0 g of sample and spread to clean Petri dish. Check product visually for colour. Observation: ____________________________________________ Acceptance criteria: A white, crystalline powder. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 2.0 | Solubility: |  |
| --- | --- | --- |
| Procedure: Take about 1.0 g of sample into 10 ml of water and check solubility. Observation: ____________________________________________ Acceptance criteria: Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 3.0 | Identification: |  |
| --- | --- | --- |

| A. | By IR Spectrophotometry: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| B. | By Chemical test: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 4.0 | Appearance of solution: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely colored than reference solution YS7. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 5.0 | pH: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: 5.9 to 6.3 Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 6.0 | Chlorides: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 100 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 7.0 | Heavy metals: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 10 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 8.0 | Sulphated ash: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 0.1% Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 9.0 | Loss on drying: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 0.5% Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 10.0 | Assay (On dried basis): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: 98.5% to 101.5% Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 11.0 | Sieve analysis: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: As per customer requirement Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 12.0 | Bulk Density: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: As per customer requirement Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 13.0 | Elemental impurities by AAS (As per ICH Q3D): |  |
| --- | --- | --- |

| A. | Lead (Pb): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| B. | Arsenic (As): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| C. | Mercury (Hg): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.1 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| D. | Cadmium (Cd): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| E. | Cobalt (Co): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| F. | Vanadium (V): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| G. | Nickle (Ni): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| H. | Lithium (Li): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| I. | Antimony (Sb): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| J. | Copper (Cu): |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit NMT 0.5 µg/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| K. | Foreign matter: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent in 5 gm sample Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| L. | Ethylene oxide: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 0.1 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| M. | OVI/Residual solvent: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: As per ICH guideline Q3C(R8) Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 14.0 | Bacterial endotoxin test: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Lowest detection limit 5 EU/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 15.0 | Pathogen in general: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 16.0 | TAMC: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 10³ CFU/ g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 17.0 | TYMC: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: NMT 10² CFU/ g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 18.0 | Escherichia coli: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 19.0 | Salmonella: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 20.0 | Pseudomonas aeruginosa: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 21.0 | Staphylococcus aureus: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| 22.0 | Coliforms: |  |
| --- | --- | --- |
| Observation: ____________________________________________ Acceptance criteria: Should be absent Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

Revision History:

| Protocol No. | Revision Made | Ref. CC No. | Effective Date |
| --- | --- | --- | --- |
| SPEC/FG00038/01(R-00) | New specification | ------- | 21 APR 2021 |
| SPEC/FG00038/01(R-01) | Format revised as per revised SOP QC/12.Some contents revised as per FSSAI requirement. | CC/2022/003 | 01 JUN 2022 |
| SPEC/FG00038/01(R-02) | Elemental impurity by AAS added. Ethylene oxide test added. | CC/2023/010 | 01 JUN 2023 |