| 1. Product Name |  | Glycine |  |
| --- | --- | --- | --- |
| 2. Technical Name |  | 2-aminoethanoic acid. |  |
| 3. Composition |  | 100 % |  |
| 4. Origin / Source |  | Synthetic |  |
| 5. Method of Production |  | Chemical synthesis |  |
| 6. Packing | Primary | 400 GSM LDPE polybag/liner |  |
|  | Secondary | HDPE/PAPER/PAPER HDPE BAGS & PAPER /HDPE drums |  |
| 7. Shelf Life of Raw Materials |  | 5 years |  |
| 8. Storage Conditions |  | Preserve in Well-closed container at room temperature. |  |
| 9. Special Handling Requirements |  | N/A |  |
| 10. Sampling Plan |  | 100 % |  |
| 11. Visual Inspection Parameters | Parameters |  | Acceptance Criteria |
|  | Description |  | A white, crystalline powder. |
| 12. Testing Parameters | Parameters |  | Acceptance Criteria |
|  | Solubility |  | Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. |
|  | Identification | A) By IR | IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. |

|  | Parameters |  | Acceptance Criteria |
| --- | --- | --- | --- |
|  |  | B) By Chemical test | The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. |
|  | Appearance of solution |  | A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely colored than reference solution YS7. |
|  | pH |  | 5.9 to 6.3 |
|  | Chlorides |  | NMT 100 ppm |
|  | Heavy metals |  | NMT 10 ppm |
|  | Sulphated ash |  | NMT 0.1% |
|  | Loss on drying |  | NMT 0.5% |
|  | Assay (On dried basis) |  | 98.5% to 101.5% |
|  | Additional Tests (on customer request, this is not part of typical specification) |  |  |
|  | Sieve analysis |  | As per customer requirement |
|  | Bulk Density |  | As per customer requirement |
|  | Elemental impurities by AAS (As per ICH Q3D) |  |  |
|  | Lead (Pb) |  | Lowest detection limit NMT 0.5 µg/g |
|  | Arsenic (As) |  | Lowest detection limit NMT 0.5 µg/g |
|  | Mercury (Hg) |  | Lowest detection limit NMT 0.1 µg/g |
|  | Cadmium (Cd) |  | Lowest detection limit NMT 0.5 µg/g |

|  | Parameters | Acceptance Criteria |
| --- | --- | --- |
|  | Cobalt (Co) | Lowest detection limit NMT 0.5 µg/g |
|  | Vanadium (V) | Lowest detection limit NMT 0.5 µg/g |
|  | Nickle (Ni) | Lowest detection limit NMT 0.5 µg/g |
|  | Lithium (Li) | Lowest detection limit NMT 0.5 µg/g |
|  | Antimony (Sb) | Lowest detection limit NMT 0.5 µg/g |
|  | Copper (Cu) | Lowest detection limit NMT 0.5 µg/g |
|  | Foreign matter | Should be absent in 5 gm sample |
|  | Ethylene oxide | NMT 0.1 ppm |
|  | OVI/Residual solvent | As per ICH guideline Q3C(R8) |
|  | Microbiological parameters |  |
|  | Bacterial endotoxin test | Lowest detection limit 5 EU/g |
|  | Pathogen in general | Should be absent |
|  | TAMC | NMT 10³ CFU/ g |
|  | TYMC | NMT 10² CFU/ g |
|  | Escherichia coli | Should be absent |
|  | Salmonella | Should be absent |
|  | Pseudomonas aeruginosa | Should be absent |
|  | Staphylococcus aureus | Should be absent |
|  | Coliforms | Should be absent |
| 13. Retained Sample Quantity |  | 30 gm |
| 14. Retained Sample Retention Period |  | Expiry + 1 year |

Revision History:

| Specification No. | Revision Made | Ref. CC No. | Effective Date |
| --- | --- | --- | --- |
| SPEC/FG00038/01(R-00) | New specification | ------- | 21 APR 2021 |
| SPEC/FG00038/01(R-01) | Format revised as per revised SOP QC/12.Some contents revised as per FSSAI requirement. | CC/2022/003 | 01 JUN 2022 |
| SPEC/FG00038/01(R-02) | Elemental impurity by AAS added. Ethylene oxide test added. | CC/2023/010 | 01 JUN 2023 |